Appendix II-C – Environmental licensing procedure in Italy, Cuba, Spain and Colombia

|  | Stage | Description | Parties involved |
| --- | --- | --- | --- |
| ITALY | Susceptibility verification (Verifica di assoggettabilità) | The decision whether a project of the attachments II bis and III of DL16/2017 must follow a full EIA process is based on the information supplied by the petitioner of the project, regarding its environmental effect, the significance of the impacts, the feasibility of technical and economic measures to reduce impacts and the social acceptance received during a period of public advertising of the proposal. Attachments I of DL16/2017 sets the criteria for the screening. | Environmental licensing authority + public participation |
|  | Private audience | During a meeting with the petitioner, the environmental authority notifies details about the content of the environmental impact study (EIS) and the impact evaluation methodology. Such indications incorporate the observations received from the public during the period of public advertising, when the screening takes place. |  |
|  | Program and design Frameworks | Within the preparation of the EIS, the petitioner must integrate a section that indicates the coherence of the activities proposed in the project with the existing pacification of the area (urban/territorial plans). Another section of the EIS must contain a description of the project or activity proposed. | Petitioner |
|  | Environment Framework | Within the preparation of the EIS, the petitioner document in this section the Identification of components, factor and their interactions that represent an eventual critical issue. It is registered the quality level of environmental components of interest, according to the type of intervention, and the use of resources. Contains the identification, estimation and evaluation of environmental impacts, an alternative analysis and the definition of environmental management and monitoring along with emergency measures. |  |
|  | Judgement of compatibility | Gathering of information and observations from the concerned population after the publication in newspapers and website of the environmental authority of a non-technical synthesis of the project prepared by the petitioner. A public audience can be called among the petitioner and the individuals of the public that submit observations or concepts that require adjustments to the EIS. Consultation and gathering of concepts from concerned institutions over the adjusted EIS. Evaluation of the EIS by a technical committee of every relevant aspect (EIA commission). | Environmental licensing authority + public participation + Concerned institutions |
|  | Decree of environmental compatibility | Formulation of a concept on the environmental compatibility of the interventions, based on a cost-benefit balance of every relevant aspect, through an Authorization Decree. At the regional level, this decision tends to result from a meeting, called Conferenza di servizi, that gathers invited representatives of the concerned institutions with the petitioner. | Environmental licensing authority |
|  | Compliance verification | During the construction, operation and closing of the project, there must be applied the prescriptions of the Authorization Decree regarding the management measures and the monitoring environmental conditions. Petitioner report the advances in such prescription in the time specified and toward the institutions assigned for the verification in the Decree. | Petitioner + Concerned institutions |

|  | Stage | Description | Parties involved |
| --- | --- | --- | --- |
| CUBA | Micro-localization | The petitioner request to the Institute of physic pacification a Macro and then micro-localization report of the location of interest for the project or activity, according to the territorial and urban plans (eg. POT). The micro-localization report contain information regarding the terrain where the investment is to be emplaced, a brief description of the works; the binding of the investment with social and economic development of the territory; interaction of the investment with supply systems (transportation and infrastructure, underground pipes and connections) and works required for articulating the investment; a list of urban and natural elements to be affected by the investment; applicable sectoral regulations... | Institute of physic pacification + Consultation bodies |
|  | Institutional consultation | Micro-localization report is delivered to consultation bodies for revision and issuance of an official concept (approved/denied) regarding the intervention and precisions to be considered in the environmental impact report. |  |
|  | Public consultation | Implementation of a communication program to the citizenship regarding the project characteristic and possible implications. | Petitioner |
|  | Short EIS | Preparation of a synthetic document addressed to the environmental authority. In general lines, the document contains: Separately characterization of natural components (air, water, soil and biota quality); Description of the socioeconomic context; Description of the project; ID and description of presumed environmental impacts; Program of prevention, mitigation and correction measure for the presumed impacts; Previsions after definitive shut down of the investment; Measures to mitigate the impact of accidents and disaster situations; Evidences of the public consultation; Monitoring program. |  |
|  | Short EIS review | Evaluation of the synthetic EIS and communication of decision: a) denial of the environmental license. b) approval of the environmental license without further scrutiny. c) conditional approval to the elaboration of an extended EIS. | Environmental licensing authority |
|  | Long EIS preparation* | Preparation of an extended document according to the guidelines defined by the environmental authority about the content of the EIS and methodologies to be used in the environmental evaluation. | Petitioner |
|  | Long EIS review* and Judgement | Evaluation of the extended EIS and communication of decision: a) License denied due to the manifestation of negative impacts or the evidence of a better environmental alternative to the project. b) License stopped due to incomplete, mistaken or inconsistent information supplied. The investor can correct the deficiency and/or hire another EIS (from a different certified institution) when required by the environmental authority. c) License granted. | Environmental licensing authority |
|  | Environmental inspections | An inspection is conducted before the operation of the project, but after the construction phase, to certify the compliance of the obligations imposed in the license or prescriptions transferred in the concepts of consultation bodies. Afterwards, random inspections take place in company with officers from the territorial system to verify the compliance of the obligations imposed in the approved license. | Consultation bodies (Environmental licensing authority) |
|  | Cooperative surveillance | Officers from one or several territorial systems can look after environmental obligations during an inspection without the presence of an officer from the environmental authority itself. |  |

|  | Stage | Description |  | Parties involved |
| --- | --- | --- | --- | --- |
| SPAIN | Environmental impact report | Is the concluding document from the simplified EIA procedure followed by the project listed in the attachments II of Law21/2013. Attachments II sets the criteria for the screening. |  | Overall licensing authority + Environmental licensing authority |
|  |  | In this report, the environmental authority dictates if the environmental license is rejected, approved without further scrutiny or requires following an ordinary EIA procedure. |  |  |
|  |  | The overall licensing authority presents to the environmental one a document developed by the petitioner with a description of the project, the alternatives, the impacts on components and the monitoring strategies to the management measures. |  |  |
|  |  | In the simplified EIA, the environmental authority conducts the consultation to the affected administrations and the interested public. |  |  |
|  | Public and institutional consultation | Voluntary request from the overall licensing authority for a document defining the scope, level of detail and degree of specification required for the EIS in the ordinary EIA procedure. The environmental authority consult the administrations affected and the interested public for defining the scope document |  | Environmental licensing authority + Consulting bodies |
|  | Alternatives examination | Within the preparation of the EIS, the petitioner must integrate a section that presents the mains alternative considered, including the nonintervention one, and the justification of the selected alternative |  | Petitioner |
|  | EIS Preparation | Within the preparation of the EIS, the petitioner must integrate the following sections: General description of the project, utilization of resources and waste generation; Evaluation, hopefully quantitative) of the project impacts in the natural components; Implications to the Natura 2000 habitat network; Prevention, correction and compensation measures; Environmental surveillance program. |  |  |
|  | EIS Review | Implementation of a program for public communication and consultation to the administrations affected through physical and electronic communication media. When the environmental authority has conducted the queries to the administrations affected and the public interested during the scoping of the EIS, the results of this consults can be used during the environmental licensing procedure. |  | Overall licensing authority + Consulting bodies |
|  |  | Creation and analysis of the environmental impact file with the documents of the project, EIS and the results of the public communication and administration consulting. |  | Environmental licensing authority |
|  | Environmental impact declaration | Concluding document of the ordinary EIA, indicating the approval or denial of the project or activity. When approved, the document also establishes the management measures, as well as the surveillance program of the conditions. |  |  |
|  | Surveillance | Inspections are conducted during the phases of construction and exploration to verify the compliance of environmental measures, among others, and the expected evolution of the environmental elements. Whereas new measures can be inserted or modified. |  | Overall licensing authority + Environmental licensing authority + Consulting bodies |
|  |  | In projects of national competence, the monitoring of certain prescriptions is carried out by the competent body of the autonomous community. |  |  |
|  |  | The manager of the public domain can enter to monitor the area of his competition |  |  |
|  | Follow-up reports | Some projects are required to elaborate and present reports about the compliance of corrective and compensatory measures established in the environmental impact declaration or report. These reports are not requested during the project validity, only during the first years of exploration. |  | Petitioner |
|  |  | The environmental body and the administrations consulted in the first stages of the process can also review these reports. |  | Overall licensing authority |
|  |  | The program of environmental surveillance and the checklist of compliance are made available to the public in electronic media. |  |  |
|  | Stage | Description | Parties involved |  |
| COLOMBIA | Environmental alternative diagnosis | Under request from the proponent, the environmental authority must pronounce about the need of conducting a preliminary procedure for assessing and compare different options presented by the petitioner, under which it is possible to develop a project, work or activity. Art.2.2.2.3.4.2 of Dec1076/2015 stablish kinds of project that may require such alternative diagnosis. The pronouncement may dictate either the project can follow the EIA procedure directly or must complete an environmental alternative diagnosis before selecting the alternative with which to continue the EIA procedure, as synthesized in the following stages. | Petitioner + Environmental licensing authority |  |
|  | Terms of reference | As a way of scoping the EIA, the national environmental authority develops and make public general guidelines for the preparation and execution of environmental studies. This guideline must be also used by regional authorities when conducting licensing procedures within its competences. However, the competent environmental authority must formulate terms of reference when there are nonspecific ones for a type of project or activity. | Environmental licensing authority |  |
|  | EIS preparation | The applicant must adapt the generic ToR to the particularities of the project, work or activity. The EIS must integrate the information of the National System of Environmental Information (SIAC), currently available online (http://www.siac.gov.co/) In general lines, the EIS must have the following sections: Project information; Characterization of the project's influence area; Demand for natural resources; Environmental impacts evaluation and risk analysis; Environmental management zoning of exclusion and intervention areas; Economic evaluation of the positive and negative impacts; Environmental management plan; Monitoring program; Contingency plan; Dismantling and abandonment plan; Investment plan of 1% of the project cost in works and actions that favor the hydrographic basin from where the project captures water; Compensation plan for biodiversity loss. | Petitioner |  |
|  | Environmental viability | The environmental authority conducts a documentary evaluation and a field technical visit to the project location, when applicable. Documents are verified according to the General Methodology for the Presentation of Environmental Studies, the terms of reference and the Geographical Information Delivery Guide. There can be asked the pronouncement of other entities to obtain sufficient elements of judgment. Up to two meetings with the proponent can be called by the authority to inform the sufficiency of not of the supplied documents and to ask for additional information. The proponent must adjust the environmental study according to the additional information requested to complete the analysis. An environmental public hearing may be convened during the environmental licensing process if requested or considered necessary. | Environmental licensing authority + Petitioner + Public + other entities |  |
|  | Resolution of environmental license | The feasibility or not of the environmental license is based on a technical concept. | Environmental licensing authority |  |
|  | Environmental compliance reports (ICA) | Periodic reports about the efficiency and effectiveness of the management measures implemented in relation to the environmental management plan, the monitoring program, the contingency plan, as well as the dismantling and abandonment plan, compensation plan and the 1% investment plan, if applicable. Periodicity is defined in the environmental licensing resolution (every 3, 6 or 12 months). | Petitioner |  |
|  | ICA review and inspections | Competent environmental authority conducts the follow-up of project or activities according to the criteria defined in the Environmental Monitoring Manual published. There tend to be sporadic technical visits to the project location. Scientific entities ascribed to the National Environmental System (SIAM) can support the environmental authority in the follow-up. | Environmental licensing authority |  |